Floppy Drive Pin Outs

| Pin | Name | Dir | Description |
| --- | --- | --- | --- |
| 1 | CGND |  | Chassis ground |
| 2 | CGND |  | Chassis ground |
| 3 | CGND |  | Chassis ground |
| 4 | CGND |  | Chassis ground |
| 5 | -12V |  | -12 VDC |
| 6 | +5V |  | +5 VDC |
| 7 | +12V |  | +12 VDC |
| 8 | +12V |  | +12 VDC |
| 9 | n/c | - | Not connected |
| 10 | PWM | ? | Regulates speed of the drive |
| 11 | CA0 | ? | Control line to send commands to the drive |
| 12 | CA1 | ? | Control line to send commands to the drive |
| 13 | CA2 | ? | Control line to send commands to the drive |
| 14 | LSTRB | ? | Control line to send commands to the drive |
| 15 | WrReq- | ? | Turns on the ability to write data to the drive |
| 16 | HdSel | ? | Control line to send commands to the drive |
| 17 | Enbl2- | ? | Enables the Rd line (else Rd is tristated) |
| 18 | Rd |  | Data actually read from the drive |
| 19 | Wr |  | Data actually written to the drive |

Much of this information comes from page 90 of Open-Apple Volume 1, number 11 (1985).

The following table lists all of the drive control signals for each type of controller card/disk port.

Signal   Function                 Unidisk   IIc       IIgs      Disk ][

GND      ground reference         1-4       1-4       1-3       1,3,5,7

-12V     -12 volts DC             5         5         5         9

+5V      +5 volts DC              6,16      6         6         11,12

+12V     +12 volts DC             7,8       7,8       7,8       13,15,17,19

WRPROT   write protect            10        10        10        20

PH 0-3   stepper motor phases     11-14     11-14     11-14     2,4,6,8

WREQ     write request            15        15        15        10

DRVEN    drive enable             17 (9)    17        17 (9)    14

RDDATA   read data                18        18        18        16

WRDATA   write data               19        19        19        18

EXTINT   external interrupt       -         9         -         -

3.5DISK  Apple 3.5 drive enable   -         -         4         -

HDSEL    3.5" drive head select   -         -         16        -

not connected            -         16        -         -

The UniDisk uses pin 9 to select the second drive.  Inside each UniDisk, the signal from pin 9 at the computer is connected to pin 17 of the daisy-chain drive connector. When the computer selects drive 2 by activating pin 9, the first drive passes this through and the second drive sees its enable signal on pin 17.  Thus all drives are identical.

The Disk][ controller has two drive connectors, and the same pin (14) is used on each connector to select the appropriate drive.  This signal is the only difference between the connectors - all other signals are connected in parallel.

Despite the IIgs having special functions for pins 4 and 16, they may be ignored when dealing with 5.25" drives, and treated as a UniDisk controller
(i.e. connect pin 4 to ground, and pin 16 to +5V).

The Apple 3.5 drive disconnects these signals internally, so that they will not interfere with its operation.